#include <iostream>

#include <fstream>

#include <sstream>

#include <vector>

#include <deque>

#include <array>

#include <chrono>

#include <cmath>

#include <csignal>

#include <cstring>

#include <unistd.h>

#include <fcntl.h>

#include <sys/ioctl.h>

#include <linux/i2c-dev.h>

#include <algorithm>

#include <numeric>

#include <termios.h>

#include <map>

#include <vector>

#include <deque>

#include <array>

#include <cmath>

#include <algorithm>

#include <numeric>

#include <complex>

// COMANDO DE COMPILAÇÃO

// g++ -O3 -march=native -ffast-math gait_reader5.cpp -o gait_reader8 -lm

// SEU MODELO LOCAL

#include "gait_xgb_v5_3_min_model.h"

using namespace std;

double ax;

double ay;

double az;

class Kalman {

private:

// Parâmetros de Ajuste

double Q; // Ruído do Processo (Inércia/Mudança real esperada)

double R; // Ruído da Medição (Erro do Sensor)

// Estado do Filtro

double P; // Covariância do erro da estimativa (Incerteza atual)

double x; // Estado estimado (O valor "limpo")

double K; // Ganho de Kalman

public:

// Construtor: Inicializa com R (sensor), Q (movimento) e valor inicial

Kalman(double process_noise, double sensor_noise, double estimated_error, double initial_value) {

this->Q = process_noise;

this->R = sensor_noise;

this->P = estimated_error;

this->x = initial_value;

}

// O coração do algoritmo: Atualiza a estimativa com uma nova leitura

double update(double measurement) {

// 1. Predição (Time Update)

// P = P + Q (A incerteza aumenta um pouco com o tempo)

P = P + Q;

// 2. Correção (Measurement Update)

// K = P / (P + R) (Calcula o peso: confio mais na predição ou no sensor?)

K = P / (P + R);

// x = x + K * (medida - x) (Atualiza o valor estimado)

x = x + K * (measurement - x);

// P = (1 - K) * P (Reduz a incerteza após a correção)

P = (1 - K) * P;

return x; // Retorna o dado limpo

}

};

// ========================= TERMINAL HANDLING =========================

struct termios original_termios;

// Sets terminal to raw mode (no buffering, no echo)

void setup_terminal() {

tcgetattr(STDIN_FILENO, &original_termios);

struct termios raw = original_termios;

// Disable canonical mode (line buffering) and echo

raw.c_lflag &= ~(ICANON | ECHO);

raw.c_cc[VMIN] = 0;

raw.c_cc[VTIME] = 0;

tcsetattr(STDIN_FILENO, TCSAFLUSH, &raw);

}

// Restores terminal settings to normal (canonical) mode

void restore_terminal() {

tcsetattr(STDIN_FILENO, TCSAFLUSH, &original_termios);

}

// Checks if a key has been pressed without blocking

bool is_key_pressed() {

char buf[1];

return (read(STDIN_FILENO, buf, 1) > 0);

}

// ========================= MPU6050 CONFIG =========================

#define MPU_ADDR 0x68

int open_i2c() {

int file = open("/dev/i2c-1", O_RDWR);

if (file < 0) {

perror("[ERROR] Failed to open /dev/i2c-1");

return -1;

}

if (ioctl(file, I2C_SLAVE, MPU_ADDR) < 0) {

perror("[ERROR] Failed to set I2C address (0x68)");

return -1;

}

return file;

}

void init_mpu6050(int file) {

char config[2];

config[0] = 0x6B; // Power management

config[1] = 0x00; // Wake up MPU6050

write(file, config, 2);

// 2. Configurar +/- 4g (Registro 0x1C recebe 0x08)

char acc_conf[2] = {0x1C, 0x08};

write(file, acc_conf, 2);

usleep(10000);

}

short read_word_2c(int file, int addr) {

char reg[1] = { (char)addr };

write(file, reg, 1);

char data[2];

read(file, data, 2);

short val = (data[0] << 8) | data[1];

if (val >= 0x8000)

val = -((65535 - val) + 1);

return val;

}

// ========================= SIGNAL HANDLER (FOR CTRL+C FALLBACK) =========================

bool keep_running = true;

void sigint_handler(int) {

keep_running = false;

}

const int AXES = 8;

const int FEATURES = 23;

vector<complex<double>> dft(const vector<double>& signal) {

int N = signal.size();

vector<complex<double>> X(N);

for (int k = 0; k < N; k++) {

complex<double> sum(0.0, 0.0);

for (int n = 0; n < N; n++) {

double angle = -2.0 * M_PI * k * n / N;

sum += signal[n] * complex<double>(cos(angle), sin(angle));

}

X[k] = sum;

}

return X;

}

vector<double> spectral_features(const vector<double>& signal) {

if (signal.empty()) {

return {0.0, 0.0, 0.0};

}

int n = signal.size();

double m = 0.0;

for (double x : signal) m += x;

m /= n;

vector<double> centered(n);

for (int i = 0; i < n; i++) {

centered[i] = signal[i] - m;

}

vector<complex<double>> X = dft(centered);

vector<double> spec;

for (auto& z : X) {

spec.push_back(abs(z));

}

if (spec.size() <= 1) {

return {0.0, 0.0, 0.0};

}

spec.erase(spec.begin()); // remove componente DC

double spec_sum = 0.0;

for (double x : spec) spec_sum += x;

double spec_energy = 0.0;

for (double x : spec) {

spec_energy += x * x;

}

spec_energy /= spec.size();

int idx_max = max_element(spec.begin(), spec.end()) - spec.begin();

double dom_freq = (double)idx_max / max((int)spec.size() - 1, 1);

double entropy = 0.0;

if (spec_sum > 1e-12) {

for (double x : spec) {

double p = x / spec_sum;

if (p > 1e-12) {

entropy -= p * log(p);

}

}

}

return {spec_energy, dom_freq, entropy};

}

vector<double> compute_features_full(const deque<array<double, 8>>& window) {

vector<double> feats;

const int AXES = 8;

const int FEATURES_PER_AXIS = 25;

feats.reserve(AXES * FEATURES_PER_AXIS);

auto mean_vec = [](const vector<double>& v) {

if (v.empty()) return 0.0;

double sum = 0.0;

for (double x : v) sum += x;

return sum / v.size();

};

auto std_vec = [&](const vector<double>& v) {

if (v.empty()) return 0.0;

double mean = mean_vec(v);

double var = 0.0;

for (double x : v) {

var += (x - mean) * (x - mean);

}

var /= v.size(); // ddof = 0

return sqrt(var);

};

auto median = [](vector<double> v) {

if (v.empty()) return 0.0;

sort(v.begin(), v.end());

int n = v.size();

if (n % 2 == 0) {

return (v[n / 2 - 1] + v[n / 2]) / 2.0;

}

return v[n / 2];

};

auto percentile = [](vector<double> v, double p) {

if (v.empty()) return 0.0;

sort(v.begin(), v.end());

double pos = (p / 100.0) * (v.size() - 1);

int idx = (int)pos;

double frac = pos - idx;

if (idx + 1 < (int)v.size()) {

return v[idx] * (1.0 - frac) + v[idx + 1] * frac;

}

return v[idx];

};

auto zero_cross_count = [](const vector<double>& v) {

int count = 0;

for (int i = 1; i < (int)v.size(); i++) {

if ((v[i - 1] >= 0 && v[i] < 0) ||

(v[i - 1] < 0 && v[i] >= 0)) {

count++;

}

}

return count;

};

for (int axis = 0; axis < AXES; axis++) {

vector<double> signal;

signal.reserve(window.size());

for (const auto& s : window) {

signal.push_back(s[axis]);

}

int n = signal.size();

if (n == 0) {

for (int k = 0; k < FEATURES_PER_AXIS; k++) {

feats.push_back(0.0);

}

continue;

}

vector<double> diff;

diff.reserve(max(n - 1, 0));

for (int i = 1; i < n; i++) {

diff.push_back(signal[i] - signal[i - 1]);

}

double min_val = *min_element(signal.begin(), signal.end());

double max_val = *max_element(signal.begin(), signal.end());

double amp = max_val - min_val;

int center = n / 2;

int c_start = max(0, center - 2);

int c_end = min(n, center + 3);

vector<double> center_window(signal.begin() + c_start,

signal.begin() + c_end);

double center_mean = mean_vec(center_window);

double mean_abs_diff = 0.0;

if (!diff.empty()) {

for (double d : diff) {

mean_abs_diff += abs(d);

}

mean_abs_diff /= diff.size();

}

double rms = 0.0;

double energy = 0.0;

for (double x : signal) {

rms += x * x;

energy += x * x;

}

rms = sqrt(rms / n);

energy = energy / n;

int idx_max_abs = 0;

for (int i = 1; i < n; i++) {

if (abs(signal[i]) > abs(signal[idx_max_abs])) {

idx_max_abs = i;

}

}

int idx_min = min_element(signal.begin(), signal.end()) - signal.begin();

double norm_idx_max_abs = (double)idx_max_abs / max(n - 1, 1);

double norm_idx_min = (double)idx_min / max(n - 1, 1);

double diff_center_mean = 0.0;

if (!diff.empty()) {

int dn = diff.size();

int d_center = dn / 2;

int d_start = max(0, d_center - 1);

int d_end = min(dn, d_center + 2);

vector<double> diff_center(diff.begin() + d_start,

diff.begin() + d_end);

diff_center_mean = mean_vec(diff_center);

}

feats.push_back(mean_vec(signal));

feats.push_back(std_vec(signal));

feats.push_back(median(signal));

feats.push_back(min_val);

feats.push_back(max_val);

feats.push_back(percentile(signal, 10));

feats.push_back(percentile(signal, 25));

feats.push_back(percentile(signal, 75));

feats.push_back(percentile(signal, 90));

feats.push_back(signal.front());

feats.push_back(signal.back());

feats.push_back(signal.back() - signal.front());

feats.push_back(center_mean);

feats.push_back(mean_abs_diff);

feats.push_back(std_vec(diff));

feats.push_back(rms);

feats.push_back(energy);

feats.push_back(amp);

feats.push_back(norm_idx_max_abs);

feats.push_back(norm_idx_min);

feats.push_back((double)zero_cross_count(signal));

feats.push_back(diff_center_mean);

vector<double> spec = spectral_features(signal);

feats.insert(feats.end(), spec.begin(), spec.end());

}

return feats;

}

class StabilityFilter {

private:

std::vector<int> history;

size_t window_size;

size_t index;

public:

// window_size: 10 a 20 costuma ser otimo para 100Hz

StabilityFilter(size_t size) : window_size(size), index(0) {

history.resize(window_size, 0); // Inicia com classe 0

}

int filter(int raw_prediction) {

// 1. Adiciona a nova predicao ao historico circular

history[index] = raw_prediction;

index = (index + 1) % window_size;

// 2. Conta qual classe aparece mais (Voto Majoritario)

std::map<int, int> counts;

int max_count = 0;

int mode = raw_prediction;

for (int val : history) {

counts[val]++;

if (counts[val] > max_count) {

max_count = counts[val];

mode = val;

}

}

return mode; // Retorna a classe vencedora

}

};

// ========================= MAIN LOOP =========================

int main() {

signal(SIGINT, sigint_handler);

setup_terminal(); // Enable non-canonical mode

const int WINDOW_SIZE = 30;

deque<array<double,8>> window;

ofstream csv;

string filename = "log_v5_3.csv";

csv.open(filename);

// Adicionei colunas do Kalman no Header

csv << "timestamp,ax,ay,az,gx,gy,gz,roll,pitch,delay_ms,classe_prevista\n";

cout << "Starting real-time gait reader (100Hz) WITH KALMAN." << endl;

cout << "Press any key to stop." << endl;

// --- Initialize MPU6050 ---

int fd = open_i2c();

if (fd < 0) {

restore_terminal();

return 1;

}

init_mpu6050(fd);

cout << "[OK] MPU6050 initialized on /dev/i2c-1 (0x68)" << endl;

StabilityFilter stability_filter(30); // Janela de 10 predicoes para suavizar a saida

// Variáveis para guardar o erro de montagem do sensor hoje

double gyro_offset_x = 0, gyro_offset_y = 0, gyro_offset_z = 0;

double pitch_offset = 0, roll_offset = 0;

cout << "CALIBRANDO... COLOQUE O TÊNIS NO CHÃO E FIQUE TOTALMENTE PARADO POR 2 SEGUNDOS." << endl;

for (int i = 0; i < 200; i++) {

int16_t ax_raw, ay_raw, az_raw, gx_raw, gy_raw, gz_raw;

// Use a sua função real de leitura aqui:

ax_raw = read_word_2c(fd, 0x3B) / 8192.0; // ax

ay_raw = read_word_2c(fd, 0x3D) / 8192.0; // ay

az_raw = read_word_2c(fd, 0x3F) / 8192.0; // az

gx_raw = read_word_2c(fd, 0x43) ;  // gx (Mantive padrao 250dps)

gy_raw = read_word_2c(fd, 0x45)  ; // gy

gz_raw = read_word_2c(fd, 0x47) ; // gz

// Acumula os erros do giroscópio

gyro_offset_x += (gx_raw / 131.0);

gyro_offset_y += (gy_raw / 131.0);

gyro_offset_z += (gz_raw / 131.0);

// Acumula os ângulos em que o tênis está parado agora

roll_offset += atan2(ay_raw, az_raw) * 180.0 / M_PI;

pitch_offset += atan2(-ax_raw, sqrt(ay_raw * ay_raw + az_raw * az_raw)) * 180.0 / M_PI;

usleep(10000); // Espera 10ms

}

// Tira a média de tudo

gyro_offset_x /= 200.0;

gyro_offset_y /= 200.0;

gyro_offset_z /= 200.0;

roll_offset /= 200.0;

pitch_offset /= 200.0;

cout << "Calibrado com sucesso! Pode começar a andar." << endl;

// --- Initialize the local model object ---

Eloquent::ML::Port::GaitXGB model;

while (keep_running) {

// --- TERMINATION CHECK (Any key press) ---

if (is_key_pressed()) {

cout << "\n[INFO] Key pressed. Stopping loop." << endl;

keep_running = false;

break;

}

// ------------------------------------------

// --- 1. CONFIGURAÇÃO (TUNING) ---

// R (Sensor Noise): Calcule a variância com o sensor parado

double R_accel = 0.3;

double R_gyro = 0.3;

// Q (Process Noise): Ajuste fino (0.001 = muito liso/lag, 0.1 = rápido/ruidoso)

double Q_gait = 0.001;

Kalman kalmanRoll(0.01, 0.1, 1.0, 0.0);

Kalman kalmanPitch(0.01, 0.1, 1.0, 0.0);

// --- 2. INSTANCIAÇÃO DOS 6 FILTROS ---

// Sintaxe: KalmanFilter(Q, R, ErroInicial, ValorInicial)

Kalman kf_ax(Q_gait, R_accel, 1.0, 0.0);

Kalman kf_ay(Q_gait, R_accel, 1.0, 0.0);

Kalman kf_az(Q_gait, R_accel, 1.0, 0.0); // Z geralmente tem a gravidade (~9.8 ou 1g)

Kalman kf_gx(Q_gait, R_gyro, 1.0, 0.0);

Kalman kf_gy(Q_gait, R_gyro, 1.0, 0.0);

Kalman kf_gz(Q_gait, R_gyro, 1.0, 0.0);

auto last_time = chrono::steady_clock::now();

array<double,8> clean_data{}; // Para armazenar os dados filtrados

while (keep_running) {

if (is_key_pressed()) {

cout << "\n[INFO] Key pressed. Stopping loop." << endl;

keep_running = false;

break;

}

auto t1 = chrono::steady_clock::now();

array<double,8> sample{};

// --- LEITURA AJUSTADA PARA +/- 4G (DIVIDIR POR 8192.0) ---

sample[0] = read_word_2c(fd, 0x3B) / 8192.0; // ax

sample[1] = read_word_2c(fd, 0x3D) / 8192.0; // ay

sample[2] = read_word_2c(fd, 0x3F) / 8192.0; // az

sample[3] = read_word_2c(fd, 0x43) / 131.0;  // gx (Mantive padrao 250dps)

sample[4] = read_word_2c(fd, 0x45) / 131.0;  // gy

sample[5] = read_word_2c(fd, 0x47) / 131.0;  // gz

ax = sample[0];

ay = sample[1];

az = sample[2];

// Calcular DT para o Kalman

double dt = chrono::duration<double>(t1 - last_time).count();

last_time = t1;

double raw_roll = atan2(ay, az) * 180.0 / M_PI;

double raw_pitch = atan2(-ax, sqrt(ay * ay + az * az)) * 180.0 / M_PI;

// 3. Passa os ângulos brutos pelo Filtro de Kalman

double roll_k = kalmanRoll.update(raw_roll);

double pitch_k = kalmanPitch.update(raw_pitch);

sample[6] = roll_k;

sample[7] = pitch_k;

// --- 4. FILTRAGEM (O "Grosso" do trabalho) ---

clean_data[0] = kf_ax.update(sample[0]);

clean_data[1] = kf_ay.update(sample[1]);

clean_data[2] = kf_az.update(sample[2]);

clean_data[3] = kf_gx.update(sample[3]) * 0.01;

clean_data[4] = kf_gy.update(sample[4]) * 0.01;

clean_data[5] = kf_gz.update(sample[5]) * 0.01;

window.push_back(sample);

if ((int)window.size() > WINDOW_SIZE)

window.pop_front();

int predicted = -1;

if ((int)window.size() == WINDOW_SIZE) {

vector<double> feats = compute_features_full(window);

// --- CALL LOCAL C++ MODEL ---

if (feats.size() == (AXES * FEATURES)) {

float input_features[AXES * FEATURES];

for (int i = 0; i < AXES * FEATURES; ++i) {

input_features[i] = (float)feats[i];

}

predicted = model.predict(input_features);

predicted = stability_filter.filter(predicted); // Aplica o filtro de instabilidade

}

}

auto t2 = chrono::steady_clock::now();

double delay_ms = chrono::duration<double, milli>(t2 - t1).count();

auto now_ms = chrono::duration_cast<chrono::milliseconds>(

chrono::system_clock::now().time_since_epoch()).count();

csv << now_ms;

for (int i = 0; i < 8; ++i) csv << "," << clean_data[i];

// Salvando Kalman no CSV

csv << "," << delay_ms << "," << predicted << "\n";

cout << "ax:" << clean_data[0]

<< " ay:" << clean_data[1]

<< " az:" << clean_data[2]

<< " gx:" << clean_data[3]

<< " gy:" << clean_data[4]

<< " gz:" << clean_data[5]

<< " | Class: " << predicted

<< " | Delay: " << delay_ms << " ms" << endl;

// Exibindo Kalman na tela

cout << " | Dly: " << delay_ms << "ms\r" << flush;

usleep(10000); // 10 ms between reads (100 Hz)

} // CLOSES while (keep_running)

// --- CLEANUP ---

close(fd);

csv.close();

restore_terminal(); // Restore terminal settings

cout << "[INFO] Stopped gracefully. Terminal restored.\n";

return 0;

}}